Project 702

Internet Protocol version 6 - new opportunities for the European PNOs

Deliverable 2

IP over NBMA Network - Conclusions of P702

Suggested Readers:

- Engineers involved in planning Internet services for PNOs

For full publication

June 1998

The EURESCOM PARTICIPANTS in Project P702 are:

FINNET Group

BT

TELECOM ITALIA S.p.a.

Koninklijke PTT Nederland N.V.

Telecom Eireann

Telia AB

This document contains material which is the copyright of certain EURESCOM PARTICIPANTS, and may not be reproduced or copied without permission

All PARTICIPANTS have agreed to full publication of this document.

The commercial use of any information contained in this document may require a license from the proprietor of that information.

Neither the PARTICIPANTS nor EURESCOM warrant that the information contained in the report is capable of use, or that use of the information is free from risk, and accept no liability for loss or damage suffered by any person using this information.

This document has been approved by EURESCOM Board of Governors for distribution to all EURESCOM Shareholders.

Preface

(prepared by the EURESCOM Permanent Staff)

The Internet has appeared as a great challenge to the Telecommunications world. This Project examines how to combine the best of two worlds: the Store and Forward IP world and the Circuit Switched Telephony world. Its aim is to overcome the drawbacks of IP by providing the means to offer network performance and control over the offered Quality of Service. It investigates the potential of IP over public switched networks in conjunction with both IP version 4 and IP version 6 and will run two experiments to validate this.

The Project started in January 1995 with a duration of a year and has an overall budget of 70 MM. The six Participants of the Project are AF, BT, IT, NL ST and TI. The Project is led by Henk Groen from NL.

This is the second of two Deliverables of the Project titled “IP over NBMA network - Conclusions of P702”. the first Deliverable titled: “Preparation of the Experiments” contained the specification on which the results of this Deliverable are based.

The Deliverable describes the approach of the Project to allow Internet users to themselves make a choice that is a trade-off between Quality of Service and cost. It elaborates on the demonstrator implemented for the Project and provides guidelines to PNOs on how they can exploit their existing strengths and know-how in the traditional Telecoms area for Internet.

Executive Summary

This is the second Deliverable of EURESCOM Project P702 and contains the final results made available for public use. It is aimed at engineers involved in planning Internet services for the PNOs, and particularly, if they need to offer Internet services with guaranteed Quality of Service or guaranteed bandwidth. The approach was successfully presented with demonstrations at EURESCOM and at the CeBIT fair, where considerable attention was attracted. The concepts presented were seen as being at the front edge of innovation.

The Deliverable describes the approach of the Project to allow Internet users to themselves make a choice that is a trade-off between Quality of Service and cost. This choice is then translated into a choice of networks and bandwidth. It elaborates on the demonstrator implemented for the Project and provides guidelines to PNOs.

The Deliverable can provide PNOs with additional ideas on ways to combine the best of two worlds: the store-and-forward Internet world and the switched world of traditional Telecoms. It shows how the traditional strengths of operators may be exploited. Both existing networks, such as (narrowband) ISDN and current and future advanced networks, such as ATM, can be integrated in the approach, allowing an immediate start with a potential for the future.

The basis for the efforts in P702 were QoS on demand by alternative trunking, thereby making use of circuit switched network technologies and IPv6. The Project has shown with experiments and actual implementations that QoS on demand is feasible and may be implemented now. In addition,  the approach followed allows a smooth transition between current and future (virtual) circuit switched techniques. The Deliverable describes the application environment and the demonstrator set-up. Telebit and the University of Lancaster supported the Project as subcontractors to achieve the results and especially for the implementations.

The promising results of the Project suggest following up these ideas into new directions, such as multicast and scaling up to larger networks, including experimentation with a larger number of users. A further Project could bring the concepts behind the approach to a level that would allow PNOs to directly implement an “Internet with QoS Choice”, providing customers with options and entering the market of real-time Internet services.

List of Authors

Henk Groen (NL)

Arto Juhola (AF)

Pasi Kyheroinen (AF)

Ivano Guardini (IT)

Claudio Franco (IT)

George Tsirtsis (BT)

Martin Peck (Telebit)

Herluf Hansen (Telebit)

Table of Contents

Preface

Executive Summary

List of Authors

Table of Contents

Glossary

1 Introduction

2 Market scenarios

2.1 Introduction

2.2 New service requirements

2.3 Network requirements

2.4 Define the network architecture

2.5 Envisaged use, suggestions  for marketing people

3 The P702 Telebit demonstrator (experiment 2)

3.1 Introduction

3.2 The P702 Telebit demonstrator

4 Conclusion and recommendation

4.1 Generic

4.2 Technical

Appendix 1:  Explanation of the demonstrator

Glossary

| ADSL | Asymmetric Digital Subscriber Line |
| --- | --- |
| CPN | Customer Premises’ Network |
| DNS | Directory Naming Service |
| IETF | Internet Engineering Task Force |
| IP | Internet Protocol |
| ISP | Internet Service Provider |
| MLPS | Multi-layer Protocol Switching |
| MPLS | Multi-protocol Label Switching |
| NARP | Next-address Resolution Protocol |
| NBMA | Non-broadcast Multiple Access |
| NHRP | Next-hop Resolution Protocol |
| NHS | Next-hop Server |
| PNO | Public Network Operator |
| PSDN | Public Switched Digital Network |
| PSTN | Public Switched Telephony Network |
| QoS | Quality of Service |
| RSVP | Resource Reservation Protocol |
| UNI | User Network Interface |
| VoD | Video on Demand |
| WFQ | Weighted Fair Queuing |

1	Introduction

The current world-wide Internet networks are based on the protocol IPv4 (Internet Protocol version 4).  Drawbacks of these networks are:

- traffic management is not possible (currently more than 66,000 networks involved)

- end-to-end bandwidth cannot be guaranteed (Store and Forward transport, CBR services not guaranteed)

- security and address space are limited

- network performance and quality of service (QoS) control are undefined

Notwithstanding these drawbacks, usage is increasing and, from a technical point of view, almost all known telecommunication services and applications can be offered over this network. Examples are:

- e-messaging services

- telephony services

- audio services

- video (still picture and moving picture) services

- retrieval and conference services

The next version of IPv4 is on its way (IPv6) and is now being tested.  This version will then provide additional support concerning the following problems of the current version 4:

- “unlimited” address space (665,570,793,348,866,943,898,599 addresses per square meter of earth surface)

- security (authentication, integrity and confidentiality)

- network performance and QoS  control (traffic priority)

The aim of Project P702 was to investigate network scenarios and specify building blocks, which combine the best of two worlds: the Store and Forward IP world and the Circuit Switched Telephony world. In particular, the building blocks defined should be seen to add additional and complementary capability to the existing leased line based Internet infrastructure. The functional architecture and the functions specified should be applicable to any public switched network.

Summarising: P702 was founded on three elements:

- QoS on demand (enabling the transport of all kind of services for those who want to pay for it)

Chapters 2 and 3 cover this subject.

- Alternative trunking (enabling the use of existent and future switched network infrastructures)

Chapters 2 and 3 deal with this subject.

- IPv6 (and IPv4) (enabling a smooth transition to an unlimited IP network technology).

The advantages of IPv6 compared to version 4 are sufficiently outlined in P702 Deliverable 1 (chapter 5). IPv6 offers many additional features over IPv4 but few are directly relevant to QoS. The IPv6 flow label and options headers are the only two features of IPv6 which offer a straightforward means by which a QoS scheme can be implemented. RSVP and IP switching do not require IPv6, but both are examples of how the improved IPv6 header can be used to support emerging QoS-driven technologies. P702 made the choice to use the flow label of IPv6 for the identification of packets for a specific flow.

Two experiments were defined by P702: experiment 1 was experimenting IPv6 and using ISDN as back-up, triggered in the Decision Point by traffic characteristics, measured in the host. This experiment was done by participants of P702. This experiment was Project internal and served to get experience with IPv6. This Project internal experiment will not be reported in this Deliverable for public use. One of the main findings is that IPv6 protocol stack can be established on both UNIX and LINUX equipped workstations by loading the proper files. Tunnelling over the IPv4 network works well.

The second experiment was specified in co-operation with Telebit; in addition, the University of Lancaster produced the proper software for the demonstration of video material over an IPv6 network. This demonstrator is reported in chapter 3.

Finally chapter 4 contains the conclusions and the recommendations.

2	Market scenarios

2.1	Introduction

The way to design networks or new network functionalities is, in principle, the following (P702 Deliverable 1):

- New service requirements

- Assess the market potential

- Define network requirements

- Define the network or network functionalities

- Decision based on expected return on investments

- Trials

Item 2) is not a subject of an EURESCOM Project; however, we would like to make a choice for the residential user/small offices. Item 1 should be considered; this is necessary in order to follow a service driven approach to the design of the network and not lose the contact to the end-user requirements.

2.2	New service requirements

The present services offered over the Internet can be classified in 3 categories:

- Non-real time, e.g.

- E-mail + attachments

- File Transfer

- WWW

- Real-time, e.g.

- audio

- video

- Service mix

In contacts with INTUG-Europe there was and is one general, dominant requirement by the user:

freedom of choice

At present, the user can choose the access provider, but having done that he has to accept the service levels offered by the access provider and the core network.

P702 did not assess the market potential, but based on some views and extrapolation of the current situation the table depicted in the following page was derived.

| Service | Present QoS | Possible enhancement | Network requirement |
| --- | --- | --- | --- |
| E-mail | sufficient | - guaranteed delivery time - registered mail - security | guaranteed bandwidth on demand N/A N/A |
| attachment | insufficient too many standards, nuisance to user | agreement on coding, automation in hosts | N/A |
| FTP | reasonable | guaranteed delivery time | guaranteed bandwidth on demand |
| WWW | questionable | congestion free access | guaranteed bandwidth on demand |
| Real-time | insufficient |  | guaranteed bandwidth on demand, delay |
| Service mix | depends on service |  | guaranteed bandwidth on demand, delay |

From this table we can conclude that offering the end-user bandwidth on demand and a variety of choices (for example ranging between 1 kbit/sec and  2 Mbit/sec) we will have covered the new service requirements. In addition, the network should be able to identify the usage in order to produce a correct bill.

2.3	Network requirements

It is very useful to distinguish two network parts:

- access (between host/CPN and first router) shared by one or a few users

- core (between the first and the last router), shared by many users

The bandwidth of the access network is limited by:

- the applied modem

- the ISDN network used for the dial-up

- the capacity of the leased line

- the capacity of the transmission technology (ADSL)

- the capacity of the mobile access (e.g. GSM)

Here there is a direct trade-off between costs and performance and it is up to the user to make the choice.  This area is not the concern of P702. There is also the problem of the holding time in the local switch, but again this is a matter of tariffing and re-dimensioning of the local switch.

The bandwidth of the core network available to one user  is limited by:

- the number of users of the network

- the transmission capacity and delay between the routers

Concerning this part of the network the user has no choice. The main problems are the (total) bandwidth, the delay (not directly related to each other) and the allocation of it.

2.4	Define the network architecture

In the previous sections the interest of the end-user was addressed. A PNO must satisfy the end-user requirements for quality and capacity at minimal cost.

The approach taken by P702 is to offer a circuit oriented on-demand by pass to the end-user, based on his QoS requirements. This approach ensures a guaranteed bandwidth and a guaranteed delay.

An alternative approach is to guarantee a bandwidth/delay for all users by increasing the requirements to the core bandwidth in a generic way. However, this approach will not guarantee the bandwidth to one individual user at a certain moment. These two cases should really be distinguished, because they relate to  different approaches:

- generic bandwidth increase in the core network (for example by techniques as tag switching, IP switching, which optimise the traffic flow on an aggregate basis)

- guaranteed bandwidth on demand for a (limited) group of users

Alcatel Telecommuncation Review Quarter 4 1997 gives a good overview of the different techniques and approaches. The following table highlights the essential differences, with the entry P702/Telebit being empty in the original Alcatel publication, a vacancy that has now been filled by P702.

Table 2.1

The weakness indicated in the Alcatel publication is network utilisation. This howeverdepends on the way this feature is implemented. The network with additional quality can be made available to the users based on market dynamics. The mapping table in the decision point can be made to depend on factors such as demand and time of the day.

2.5	Envisaged use, suggestions  for marketing people

The implemented  QoS on demand feature can be offered to the user in various ways. The following list is not exhaustive:

- flat rate subscription

- options for spontaneous purchase (entry of payment details)

- option to upgrade quality spontaneously (if for instance ambient traffic volume increase), tear down messages, and new reservation established

- facilitates grown in PUSH/PULL services

3	The P702 Telebit demonstrator (experiment 2)

3.1	Introduction

The Project has produced a Cut-through Reference Model that exploits the general characteristics and requirements of a dynamic bandwidth on demand service. The model identifies the main elements of the architecture that are common in all scenarios (Figure 3.1). The main elements are:

Figure 3.1: The Cut-Through Reference Model

- CLNS: Connection-less Network such as Public Internet or Private intranets.

- Non-Broadcast Multiple Access Networks (NBMA): currently we take into account ISDN, and ATM.

- Decision Point: is the machine that has the hardware and software ability to route packets to different types of networks (CLNS, NBMA).

- Cut-through Data: is the point at which all the relevant static or dynamic information is kept or calculated.

- Policy: is the information about who is allowed to use what, when, with what characteristics, for how long and at what price.

- Flow Specification: is the information about the specific characteristic of a cut-through (bandwidth, delay etc.).

- Autonomous System (AS): is a collection of machines (router and hosts) that share one Decision Point and one set of associated Cut-through Data. Examples would be PNO/ISP, customer base.

Decision making belongs to the user but the Decision Point is in the network. Thus, the user must signal his flow requirements to the Decision Point. IETF RSVP is used for this purpose in conjunction with an IPv6 QoS routing table based on subscription information that enables the requested flow requirement to be mapped to the appropriate network type and bandwidth.

3.2	The P702 Telebit demonstrator

The figure 2 shows the EURESCOM P702 bandwidth-on demand Demonstrator set-up. The hosts H1, H2 and H3 are all connected to Decision Points DP1, DP2, DP3 respectively through Ethernet. The route through DP2 represents an IP Internet. ATM switch represents an ATM network, while ISDN1 and ISDN2 are the connection to an ISDN network. Through the ISDN network an IPv6-in-IPv4 tunnel is set up, in order to forward IPv6 traffic over the IPv4 ISDN boxes.

Figure 4.2: IPv6 Bandwidth on Demand Demonstrator

Firstly, users subscribe to this bandwidth-on-demand service, and subscription information is stored at the Decision Points as policy information.

When starting an application, the user selects via a graphical interface (or the application selects automatically) the QoS parameters for the flow transmitted by the application. This is signalled to the Decision Point using an RSVP PATH message. It is then routed according to the next-hop information found in an IPv6 QoS routing table. A Flow ID (based on source address and the IPv6 Flow Label) is then used to register the flow in reservation table, enabling subsequent data packets for which a Flow ID match is found to bypass a full router table look-up.

The mapping of QoS parameters to selection of bearer network at Decision Points is subscription specific and under the control of the PNO. This implicitly is a choice of bearer network, and the required network bandwidth.

Users are notified of rejected requests for QoS. The signalling of flow requirements (and reject/success results) are signalled by RSVP. Accounting information is collected at Decision Points and transmitted as account records to a Network Management Centre where they are logged.

In general as much as possible is designed according to available or emerging IETF RFCs, ATM Forum specifications and ITU recommendations. In particular the signalling between host and Decision Point of user/application selections is done by means of IETF RSVP over IPv6, which is currently on the standards track. Encapsulation of IPv6 for different media is also according to IETF drafts/RFCs. The ATM SVC service is accessed according to ATM Forum UNI 3.1

What goes beyond the framework of the above specifications is primarily the concept of Decision Points where assignment of routes of IPv6 packets in a flow is based not only on Destination and Source IPv6 address, but also on a EURESCOM defined interpretation of other fields in the RSVP PATH message. This interpretation, together with policy information, is configured in the Decision Points by means of network management, and forms the basis of deciding whether to assign a route for a flow via ISDN or ATM SVC networks or normal IPv6 Internets.

4	Conclusions and recommendations

4.1	Generic

QoS on user demand is a feasible,  attractive (from a user perspective) and required feature for an all-services IP network. The P702 way of implementation is straightforward,  does not require  additional hard-ware developments and enables the monitoring of the usage of this feature (registration of RSVP usage). All current and future NBMA technologies are applicable; this enables network operators to have a flexible and economic transition. IPv6 is attractive in the implemented scenario but not really required (there alternatives to the flow label). It is very attractive to widen the usage of the P702 concept by applying it on a Europe-wide scale.

4.2	Technical

The following activities are recommended to enhance the P702 concept:

Non-IPv6 related:

- Flow aggregation: It is possible to carry more RSVP flows over the same ATM SVC channel.

- Use of WFQ to control any L2 link, and not just ISDN, and to reserve resources on LANs.

- Dynamic cut-through policy and decision mapping (e.g. based on cost/availability).

- Frame Relay as NBMA

IPv6 related:

- QoS multicast routing: RSVP is optimised for multicast, and is only currently implemented in a unicast mode. This would require the development of a multicast application, multicast routing support of IPv6 (e.g. PIM), integration of RSVP with multicast routing, development of a multicast QoS routing table and the mapping of multicast to ATM multicast channels.

- Integration with a differential services approach making use of the IPv6 Traffic Class field.

- Comparison of IPv4 and IPv6 style flow classification.

- Native IPv6 over ISDN. Currently no native ISDN support is provided. A Project on integrated ISDN support could also include MPLS integration

- Support of NHRP. With the aim of better integrating IPv6 and ATM, support of NHRP would allow the optimisation of RSVP to be initiated

Appendix 1:  Explanation of the demonstrator

Stepwise description (based on a video on demand service, running in a server):

- Customer selects video from menu and clicks on PLAY

- Client in host signals to the server to start the video

- Video server starts and assigns random flow label. This is combined with the destination address > unique flow idea

- Client initiates RSVP sequence specifying required BW

- Server sends RSVP PATH_MESSAGE, signalling the BW requirements to the first Decision 	Point (DP1)

- PATH_MESSAGE routed using QoS Routing Table mapping BW to port no.

Source Prefix		Dest Prefix		Bandwidth		Interface

max 	min	`

xxxxxxx		xxxxxx		64k	8k		Internet (Best Effort)

xxxxxxx		xxxxxx		2M	64k		ISDN

xxxxxxx		xxxxxx		8M	2M		ATM

- Path through network installed

- Client then sends reservation message back through network following the initial RSVP PATH_MESSAGE

- Remote decision point then sets up connection and installs reservation

- All subsequent packets with identical Flow ID are identified and forwarded using a Reservation Table.

Notes:

Service provision under the control of policy and subscription information held by a PNO controlled Network Management Centre (NMC)

Full accounting facilities available

Quality can be changed while watching the video